TY125-D

SERVICE MANUAL

Foreword

This service manual contains the technical data of each component inspection and repair for the TY125-D motorcycle. The manual is shown with illustrations and focused on "Service Procedures", “Operation Key Points”, and "Inspection Adjustment" so that provides technician with service guidelines.

If the style and construction of the motorcycle, TY125-D, are different from that of the photos, pictures shown in this manual, the actual vehicle shall prevail. Specifications are subject to change without notice.

Zhejiang Tianying Locomotive Co., Ltd.

- I.General information

- 01-The main technical parameters………………………3

- 02-Lubrication system specifications……………………4

- 03-Crankshaft, piston and cylinder specifications………4

- 04-Cylinder head and valve specifications………………5

- 05-Torque value……………………………………………6

- 06-Electrical device location map…………………………8

- 07-Service precautions……………………………………9

- 08-Troubleshooting………………………………………15

- 09-Lubrication points……………………………………19

- II.Maintenance information

- 10-Air cleaner……………………...……………………...20

- 11-Inspection and change of the engine oil… …………21

- 12-Steering handle top bearing……… …………………22

- 13-Cushion………………………………………………21

- 14-Drive chain……………………………………………23

- 14.1-Check the chain…………………………………23

- 14.2-Adjust the chain…………………………………23

- 15-Schematic diagram of the braking system…………24

- 15.1-Precautions in operation…………………………25

- 15.2-Troubleshooting…………………………………26

- 15.3-Disc brake system inspection……………………27

- 15.4-Add brake fluid……………………………………28

- 15.5-Brake fluid replacement / Air-bleed………………29

- 15.6-Front brake caliper…………………………………30

- 15.7-Rear brake caliper…………………………………31

- 15.8-Brake disk…………………………………………32

- 15.9-Master cylinder……………………………………32 III.Engine maintenance

- 16-Main characteristics…………………………………35

- 17-Main technical specification…………………………35

- 18-Precautions for use…………………………………36

- 19-Regular maintenance project and schedule………38

- General information

- 01—Main technical parameters：

| Item | Technical parameters | Item | Technical parameters |
| --- | --- | --- | --- |
| L×W×Hmm | 1985×745×1075 | Engine type | K157FMI, Single-cylinder, four-stroke, Natural air cooling, vertical cylinder |
| Wheelbase mm | 1360 | Transmission type | chain transmission |
| Front wheel size | 100/80-17 | Transmission ratio | Gear： 1：31.97，2:19.795， 3:14.582,4:11.672, 5:9.732 |
| Rear wheel size | 120/70-17 | Compression ratio | 9.5:1 |
| Ground clearance mm | / | Bore diameter × stroke mm | 57*48.8 |
| Curb weight kg | 136 | Working volume ml | 124.5mL |
| Max load kg | 150 | Clutch type | Manual clutch |
| Handlebar swivel angleº | ≤48 | Lubrication method | Pressure, pressure splash combined |
| Max speed km/h | 95 | Idle speed r/min | 1700±100 |
| Economic fuel consumption | 350g/KW·h | Crankcase lubricating oil volume L | 1.26 |
| Climbing performance º | ≥18 | Lubricant grade | SF grade SAE 15W/40（crankcase） |
| Battery | 12V 7Ah | Ignition method | ECU Electronically controlled ignition |
| Fuel grade | Above RQ90 | transmission case oil volume L | / |
| Spark plug specifications | CR7HSA | Gear oil grade | / |
| Spark plug gap | 0.6～0.7 | Fuse | 15 A |
| Gasoline volume L | 13.5 | Headlamps (High Beam) | LEDs×4 31W 12V |
| Front position light | LEDs×2 2.4W 12V | Headlamp (low beam) | LEDs×3 22.8W 12V |
| Front turn signal | LED*1 1W 12V | Rear license plate lights | LED*3 0.2W 12V |
| Rear turn signal | LED*1 1W 12V | Brake (brake) light | LED*4 3.5W 12V |
|  |  | Rear tail light | LED*8 2.5W 12V |

## 02-Lubrication system specifications:	             unit: mm

| Item |  | Standard value | Maintenance threshold |
| --- | --- | --- | --- |
| Engine oil capacity | change engine oil | 0.85 L | - |
|  | after removing the engine | 1.26 L | - |
| 80W-90 Gear Oil | change gear oil | / | - |
|  | after removing the engine | / | - |
| Recommended engine oil |  | Recommended engine oil: SF15W-40 API Quality Grade: SG or higher (do not use oils marked as energy efficient on the circular API service label) | - |
| Engine oil pump rotor | Tip clearance | Not more than 0.15 (between inner and outer rotors) | 0.16 |
|  | Intermediate gap | 0.016～0.074 (inner rotor hole and shaft) | 0.084 |
|  | Gap on both sides | 0.06 to 0.12 (up and down direction) | 0.13 |

03-Crankshaft, piston and cylinder specifications:               unit: mm

| Item |  |  | Standard value | Maintenance threshold |
| --- | --- | --- | --- | --- |
| Crankshaft | Large end side clearance of connecting rod |  | 0.10～0.35 | 0.45 |
|  | Clearance between connecting rod big end bearing bush and crank pin |  | 0.097～0.127 | 0.07 |
|  | Beat |  | - | 0.05 |
| Cylinder block | Bore |  | 57.006～57.012 | 57.12 |
|  | Out-of-roundness |  | - |  |
|  | Taper |  | - |  |
|  | Flatness |  | - |  |
| Pistons, Piston Pins, Piston Rings | Piston base circle diameter |  | 57.275～57.285 | 57.19 |
|  | Pin hole diameter |  | 14.002～14.008 | 14.02 |
|  | Piston pin diameter |  | 13.994～14 | 13.98 |
|  | Piston to Piston Pin Clearance |  | 0.002～0.014 | 0.22 |
|  | Piston ring closed gap | First ring | 0.10～0.30 | 0.35 |
|  |  | Second ring | 0.20～0.40 | 0.45 |
|  |  | Oil ring (scraper ring) | 0.20～0.70 | 0.9 |
|  | Piston ring and ring groove clearance | Clearance between first ring and groove | 0.020～0.060 | / |
|  |  | Gap between second ring and groove | 0.020～0.060 | / |
| Cylinder clearance |  |  | 0.010～0.045 | 0.055 |
| Inner diameter of connecting rod small end |  |  | 14.01～14.021 | 14.121 |
| Fitting clearance between connecting rod and pin |  |  | 0.01～0.027 | / |

04-Cylinder head and valve specifications:	             unit: mm

| Item |  |  | Standard value | Maintenance threshold |
| --- | --- | --- | --- | --- |
| Electric start cylinder pressure |  |  | (1000～1200)kPa | — |
| Valve clearance |  | Intake valve | 0.06～0.08 | — |
|  |  | Exhaust valve | 0.10～0.13 | — |
| Rocker arm, rocker arm shaft | Rocker arm inner diameter | Intake/exhaust | 11.968～11.98 | 11.982 |
|  | Rocker bearing outer diameter | Intake/exhaust | / | / |
|  | Rocker arm and bearing | Intake/exhaust | / | / |
|  | Gap | Intake/exhaust | / | / |
|  | Rocker bearing inner diameter | Intake/exhaust | 11.972～11.987 | 11.962 |
|  | rocker shaft outer diameter | Intake/exhaust | / | / |
| Camshaft | Camshaft hull height | Intake | 33.676～33.776 | 33.666 |
|  |  | Exhaust | 33.463～33.563 | 33.453 |
|  | Bearing and bore clearance |  | 0.002～0.026 | 0.036 |
|  | Beat |  | / | / |
| Valve/vale catheter | Valve stem diameter | Intake | 4.975～4.99 | 4.965 |
|  |  | Exhaust | 4.955～4.97 | 4.945 |
|  | Valve guide inner diameter | Intake/exhaust | 5～5.012 | 5.022 |
|  | Valve stem to valveduct gap | Intake | 0.01～0.037 | 0.057 |
|  |  | Exhaust | 0.03～0.057 | 0.077 |
|  | Catheter height (leakage) | Intake/exhaust | 10.8～11.2 | / |
|  | Valve seat width | Intake/exhaust | 1.0～1.2 | 1.6 |
| Valve spring free length |  | Inside diameter | 32.50～32.54 | / |
|  |  | Inside diameter | 34.98～35 | / |
| Cylinder head flatness |  | 0.05 |  | 0.05 |

05-Torque values:

The torque values listed in below are for more important tightening torque values. Please see standard values for those not listed in the table.

Standard torque values for reference:

| Type | Tighten torque | Type | Tighten torque |
| --- | --- | --- | --- |
| 5 mm bolt、nut | 0.45~0.6kgf.m | 5 mm screw | 0.35~0.5kgf.m |
| 6 mm bolt、nut | 0.8~1.2kgf.m | 6 mm screw、SH nut | 0.7~1.1kgf.m |
| 8 mm bolt、nut | 1.8~2.5kgf.m | 6 mm bolt、nut | 1.0~1.4kgf.m |
| 10 mm bolt、nut | 3.0~4.0kgf.m | 8 mm bolt、nut | 2.4~3.0kgf.m |
| 12 mm bolt、nut | 5.0~6.0kgf.m | 10 mm bolt、nut | 3.5~4.5kgf.m |

Engine torque value (regular maintenance):

| Item | Quantity | Thread diameter (mm) | Torque value (N.m) | Remark |
| --- | --- | --- | --- | --- |
| Spark plug | 1 | 10 | 12～15 | / |
| Valve cover | 1 | 30 | 20～25 | Strainer cover |
| Sealing plug | 1 | 12 | 25～30 | Crankcase oil drain bolt |
| Transmission case oil drain bolt | 1 | 8 | 18～22 | With gasket |
| Transmission case refueling bolt | 1 | 8 | 18～22 | With gasket |

| Item | Quantity | Thread diameter (mm) | Torque value (N.m) | Remark |
| --- | --- | --- | --- | --- |
| Cylinder head nut | 4 | 8 | 28～32 | Oil on the nut end |
| Camshaft flapper bolt | 1 | 6 | 8～12 | / |
| Valve rocker shaft bolt | 2 | 5 | 5～9 | / |
| Oil and gas separation plate screw | 4 | ST4.2 | 2～3 | / |
| Cylinder head bolts | 4 | 6 | 8～12 | / |
| Cylinder head cover see oil bolt | 1 | 5 | 8～12 | / |
| Tensioner adjusting bolt | 1 | 6 | Hand tighten | / |
| Cylinder connecting bolt | 2 | 6 | 8～12 | / |

Cylinder head and valve:

Belt combination:

| Item | Quantity | Thread diameter (mm) | Torque value (N.m) | Remark |
| --- | --- | --- | --- | --- |
| Main drive pulley tightening nut | / | / | / | / |
| Driven pulley tightening nut | / | / | / | / |
| Left crankcase cover bolts | 10 | 6 | 8～12 | / |

Magneto:

| Item | Quantity | Thread diameter (mm) | Torque value (N.m) | Remark |
| --- | --- | --- | --- | --- |
| Magneto rotor bolts | 1 | 12 | 80～90 | Apply 1262 thread glue |
| Magneto stator bolts | 3 | 6 | 18～12 | / |
| Hall sensor bolt | 1 | 6 | 7～9 | Apply 1262 thread glue |
| Trigger fixing bolt | 2 | 6 | 8～12 | / |
| Clamping plate bolts | 2 | 6 | 8～12 | / |
| Right crankcase cover bolts | 10 | 6 | 8～12 | / |

Case and transmission system:

| Item | Quantity | Thread diameter (mm) | Torque value (N.m) | Remark |
| --- | --- | --- | --- | --- |
| Crankcase bolts | 10 | 6 | 8～12 | / |
| Crankcase positioning stud | 4 | 6 | 8～12 | / |
| Transmission case bolts | 7 | 8 | 18～22 | / |
| Pin lock nut | 1 | 8 | 18～22 | transmission case brake drum locating pin |
| Rear brake camshaft limit bolt | 1 | 6 | 8～12 | Rear brake arm |
| Press pin body tightening bolt | 1 | 6 | 8～12 | Timing chain limit plate fastening |
| Fastening bolt for speed sensor | 1 | 6 | 8～12 | / |
| Speed sensor protection board, lower | 2 | 6 | Hand tighten | / |

06-Electrical device location map:

07-Service precautions:

- Always use genuine parts and recommended oil.Using improper parts may cause damage to or

- destruction of the vehicle.

- Special tools are designed for removal and installation of component parts without damaging them. Using wrong tools may result in parts damage.

- When servicing this vehicle, use only metric tools. Metric bolts, nuts, and screws are not interchangeable with the Britain system, using wrong tools and fasteners may damage this vehicle.

- Clean the outside of the parts or the cover before removing it from the vehicle. Otherwise, dirt and deposit accumulated on the part's surface may fall into the engine, chassis, or brake

- system to cause damage.

- Wash and clean parts with high flash point solvent, and then blow dry with compressed air. Pay special attention to O-rings or oil seals because most of the cleaning agents have bad effect

- on them.

- Never bend or twist control cables to avoid unsmooth control and premature worn out.

- Rubber parts may become deteriorated when old, and be damaged by solvent and oil easily. Check these parts before installation to make sure that they are in good condition, replace if necessary.

- When loosening a component which has different sized fasteners, operate with a diagonal pattern and work from inside out. Loosen the small fasteners first. If the bigger ones are loosen first, small fasteners may receive too much stress.

- Store complex components such as transmission parts in the proper assemble order and tie them together with a wire for ease of installation later.

- Note the reassemble position of the important components before disassembling them to ensure they will be reassembled in correct dimensions (depth, distance or position).

- Components not to be reused should be replaced when disassembled including gaskets metal seal rings, O-rings, oil seals, snap rings, and split pins.

- The length of bolts and screws for assemblies, cover plates or boxes is different from one another, be sure they are correctly installed. In case of confusion, Insert the bolt into the hole to compare its length with other bolts, if its length out side the hole is the same with other bolts, it is a correct bolt. Bolts for the same assembly should have the same length.

- Tighten assemblies with different dimension fasteners as follows: Tighten all the fasteners with fingers, then tighten the big ones with special tool first diagonally from inside toward outside, important components should be tightened 2 to 3 times with appropriate increments to avoid warp unless otherwise indicated. Bolts and fasteners should be kept clean and dry. Do not apply oil to the threads.

- When oil seal is installed, fill the groove with grease, install the oil seal with the name of the manufacturer facing outside, and check the shaft on which the oil seal is to be installed for smoothness and for burrs that may damage the oil seal.

Manufacturer's name

- Remove residues of the old gasket or sealant before reinstallation, grind with a grindstone if the contact surface has any damage.

Groove

Clamp

Connector

The ends of rubber hoses (for fuel, vacuum, or coolant) should be pushed as far as they can go to their connections so that there is enough room below the enlarged ends for tightening the clamps.

- Rubber and plastic boots should be properly reinstalled to the original correct positions as designed.

Boots

- The tool should be pressed against two (inner and outer) bearing races when removing a ball bearing. Damage may result if the tool is pressed against only one race (either inner race or outer race). In this case, the bearing should be replaced. To avoid damaging the bearing, use equal force on both races.

Both of these examples can result in bearing damage.

- Lubricate the rotation face with specified lubricant on the lubrication points before assembling.

- Check if positions and operation for installed parts is in correct and properly.

- Make sure service safety each other when conducting by two persons.

- Note that do not let parts fall down.

- Before battery removal operation, it has to remove the battery negative (-) cable firstly. Notre tools like open-end wrench do not contact with body to prevent from circuit short and create spark.

- After service completed, make sure all connection points is secured.Battery positive (+) cable should be connected firstly

- And the two posts of battery have to be greased after connected the cables.

- Make sure that the battery post caps are located in properly after the battery posts had been serviced.

- If fuse burned, it has to find out the cause and solved it.	And then replace with specified capacity fuse.

Capacity verification

- When separating a connector, it locker has to be unlocked firstly；Then, conduct the service operation.

- Do not pull the wires as removing a connector or wires；Hold the connector body.

- Make sure if the connector pins are bent, extruded or loosen.

- Insert the connector completely.If there are two lockers on two connector sides, make sure the lockers are locked in properly.Check if any wire loose.

- Check if the connector is covered by the twin connector boot completely and secured properly.

- Before terminal connection, check if the boot is crack or the terminal is loose.

- Insert the terminal completely.Check if the terminal is covered by the boot. Do not let boot open facing up.

- Secure wires and wire harnesses to the frame with respective wire bands at the designated locations. Tighten the bands so that only the insulated surfaces contact the wires or wire harnesses.

- Wire band and wire harness have to be clamped secured properly.

- Do not squeeze wires against the weld or its clamp.

- Do not let the wire harness contact with rotating, moving or vibrating components as routing the harness.

- Keep wire harnesses far away from the hot parts.

Never Touch

- Route wire harnesses to avoid sharp edges or corners and also avoid the projected ends of bolts and screws.

- Route harnesses so that they neither pull too tight nor have excessive slack.

Never too tight

- Protect wires or wire harnesses with electrical tape or tube if they contact a sharp edge or corner. Thoroughly clean the surface where tape is to be applied.

- Secure the rubber boot firmly as applying it on wire harness.

- Never use wires or harnesses which insulation has been broken.Wrap electrical tape around the damaged parts or replace them.

Never clamp or squeeze the wire harness

Never clamp or squeeze the wire harness as installing other components.

- Do not let the wire harness been twisted as installation.

- Wire harnesses routed along the handlebar should not be pulled too tight or have excessive slack, be rubbed against or interfere with adjacent or surrounding parts in all steering positions.

Do you know how to set the instrument to its measurement position and the insert locations of its two probes?

Before operating a test instrument, operator should read the operation manual of the instrument. And then, conduct test in accordance with the instruction.

- With sand paper to clean rust on connector pins/terminals if found. And then conduct connection operation later.

Clean rust

08-Troubleshooting：

- A.Engine cannot be started or difficult to be started

Probable causes

Check and adjustment

Check and adjustment

The fuel supply to the fuel injector is sufficient

Press the fuel injector feed pipe and confirm whether there is fuel in the feed pipe

The fuel injector is not

Check the spark plug ignition condition

Week sparks or no spark

Sparks

Perform cylinder compression pressure test

2.Check if the fuel pipe and the vacuum

tube are blocked or not

cuvavacuumvacuum

3.Malfunction of fuel pump relay or wiring

5.The fuel strainer is blocked or not

4.Malfunction of fuel pump

2.Malfunction of throttle valve operation

1.Contaminated or clogged fuel injector

2.Malfunction of fuel pressure regulator

Wet spark plug

Dry spark plug

1.Malfunction of throttle valve operation

3.Incorrect ignition timing

2.Air sucked into intake manifold

With signs of ignition but the engine cannot be started

No ignition

5.Ignition coil is in open or short circuit

6.Malfunction of main switch

4.Malfunction of the AC Generator

3.Malfunction of ECU

2.Spark plug contamination

1.Malfunction of spark plug

2.Malfunction of cylinder valves

4.Cylinder gasket leakage

3.Worn cylinder and piston ring

5.Sand hole in the compression parts

1.Piston ring seized

Low or no compression pressure

1.Check the fuel amount in the fuel tank

Normal cylinder compression pressure

Restart the engine

Remove and check the spark plug

- B.Engine runs sluggish (Speed does not pick up, lack of power)

Fault condition

Fault condition

Accelerate gradually and
 check engine RPM

Check and adjustment

1.Clogged air cleaner

Normal

Continually drive in acceleration or high speed

Engine RPM can be increased

Abnormal compression pressure

3.Poor fuel quality

4.Abnormal ignition timing

2.Malfunction of fuel injector

1.Carbon deposit in the combustion chamber

4.Carbon deposit in the combustion chamber

3.Poor fuel quality

2.Abnormal fuel injector

1.Worn out piston or cylinder

1.Incorrect spark plug heat range

1.Remove the dirt

1.Replace the fuel injector

- 4.Valve deterioration

- 3.Sand hole in compression parts

- 5.Jammed piston ring

2.Cylinder gasket leakage

1.Worn out cylinder or piston ring

1.Malfunction of ECU

2.Malfunction of AC Generator

4.Clogged fuel injector

3.Clogged exhaust pipe

2.Poor fuel supply

No knocking

Knocking

Check cylinder compression pressure (using compression

Engine overheated

No contamination or

contamination or

Remove and check the spark plug

clogged

Not clogged

Check if the fuel injector is clogged or not

Normal compression pressure

Incorrect ignition

Correct ignition timing

Check ignition timing (Using ignition lamp)

Engine RPM cannot be increased

Check if the engine is over heated

- C.Engine runs sluggish (especially in low speed and idling)

Probable causes

Fault condition

Check and adjustment

Check ignition timing (using ignition lamp)

- Incorrect ignition timing

- (malfunction of ECU or AC）

Normal

Abnormal

Check for any air sucked in through the throttle body insulator gasket

1.Abnormal throttle body insulator gasket

2.Abnormal throttle body installation

3.Abnormal inlet pipe gasket

4.Damaged ABV pipe

Air sucked in

No air sucked in

Remove the spark plug and check the spark condition

1.Contaminated spark plug

3.Malfunction of AC Generator

2.Malfunction of ECU

4.Malfunction of ignition coil

Good spark

Poor spark

5.Open or short circuit in spark plug leads

6.Malfunction of main switch

- D.Engine runs sluggish (High speed)

Fault condition

Probable causes

Poor

Good

Check the fuel pump supply condition

Check if fuel injector clogged supply condition

Clogged

Normal

1.Replace the fuel injector

Abnormal

Normal

Check ignition timing

1.Insufficient fuel in the fuel tank

2.Pressed or clogged fuel pipe

1.Malfunction of ECU

2.Malfunction of AC Generator

Check and adjustment

- E.CLUTCH AND DRIVING PULLEY

PROBABLE CAUSES

Engine can be started but the vehicle cannot run

FAULT CONDITION

1.Worn out or damaged drive belt

2.Damaged movable drive face

3.Damaged driven face spring

4.Broken clutch weight

5.Broken drive shaft groove

6.Worn out or damaged transmission gear

1.Broken clutch spring

Engine shuts down or trembles when the vehicle is running (rear wheel rotates during engine idling)

2.Clutch outer stuck with clutch weights

3.Connection parts between clutch and shaft worn out or burned

1.Worn or deformed drive belt

Poor initial driving (poor climbing performance)

2.Worn weight roller

3.Worn driven face

4.Deformed driven face spring

5.Grease on drive belt or drive / driven

09-Lubrication points:

- Maintenance Information

- 10-Air Cleaner:

1.

Decorative cover

Remove the left-side trim cover;

- 2.拆下图示中的5颗螺栓，取下空滤器滤芯;

5 Bolts

Air filter element

3.

Remove the air cleaner element;

Air filter element

- 4.Clean air cleaner element:

- Use compressed air to remove the adhesion of dirt,if too much dirt , please new replacement.

- The air filter element is made of paper products, must not soak or cleaning by water.

Caution

- 5.Follow the opposite step to reinstall the air filter elements and components.

- 11-Inspection and change of the engine oil

11.1-Oil oil level check:

- Engine oil level Before engine start, check engine oil level;

- When checking the oil, please stand the car on the flat ground, the oil level should be between the upper limit ② and the lower limit ③;

- If necessary, add conforming oil to the oil level ② limit (oil filler ①);

Change the oil and clean the oil screen.

Engine oil capacity is 1.4 litres.

- When changing oil, the engine temperature should not be completely cooled, the motorcycle should be fixed in the main support to ensure that the oil quickly and completely discharged.

Caution

11.2-Replace oil:

The oil should be replaced when the engine is not fully cooled to release the old oil in the machine.

The oil change sequence is as follows:

1.Make the motorcycle stand upright on the flat ground;

2.Remove the filler bolt ➀;

3.Remove the oil release bolt ② at the bottom of the engine and drain the old oil inside;

4.Remove the oil filter cover ③, take out the spring, oil filter, oil filter O-ring in turn, remove the old oil, and replace the oil filter, oil filter O-ring and oil filter cover O-ring, and reinstall the parts in turn;

③ Oil filter cover, ④ oil filter cover O-ring, ⑤ spring, ⑥ oil filter, ⑦ oil filter O-ring;

5.Remove the oil filter cover and oil filter, remove the old oil, clean the oil filter cover and oil filter, replace the oil filter cover O-ring, and reinstall the parts in turn;

⑧ Oil filter screen cover, ⑨ oil filter screen cover O-ring, ⑩ oil filter screen;

(If the oil filter is damaged, please replace it immediately);

6.Then install the oil bolt ② and the oil bolt sealing gasket ⑪;

7.Pour 0.85L oil from the filler into the engine and install the filler bolt ➀;

8.Start the engine and leave it idle for a few minutes;

9.Turn off the engine and wait one minute and check that the oil level is correct.

12-Steering handle top bearing：

Caution

- Check all wires and cables if they are interfered with the rotation of steering handle bar.

Lift the front wheel out of ground；

- Turn handle from right to left alternative and check if turning is smoothly；

- If handle turning is uneven and bending, or the handle can be operated in vertical direction, then adjust the handle top bearing。

- 13-Cushion：

- Caution

- Do not ride the motorcycle with poor cushion.

- Looseness, wear or damage cushion will

- make poor stability and drive-ability.

Front cushion

- Press down the front cushion for several times to check it operation；

- Check if it is damage Replace relative parts if damage found；

- Tighten all nuts and bolts；

- Rear Cushion

- Press down the front cushion for several times to check it operation；

- Check if it is damage Replace relative parts if damage found；

- Park motorcycle with main stand；

- Turn the rear wheel forcefully and check if engine bracket bushing worn out Replace the bushing if looseness found；

- Tighten all nuts and bolts.

- Drive chain

14.1-Check the chain:

- Every 1,000 kilometers, adjust the relaxation of the transmission chain, so that the chain swings up and down between 20-30mm.

- Excessive loosening of the chain may cause serious damage to the chain or to the engine.

- The 428 HG-132 / 428HO-132 chain is recommended.

- Caution

- When the chain is adjusted to the limit, please change the chain immediately;

- If the chain is deformed, knotted, worn and other bad, and cannot be restored after maintenance, please replace the chain immediately;

14.2-Adjust the chain:

Support the motorcycle with a side bracket.

1.Loosen rear wheel nut①.

2.Loosen chain regulator lock nut②.

3.Adjust the relaxation of the chain by turning the two chain regulator bolts③.

4.The left and right sides of the rear axle shall be kept at the same scale position to ensure that the front and rear wheels are properly aligned.

5.After adjusting the chain, tighten the rear wheel nut① to the specified torque.

- Check the chain slack after tightening the rear wheel nut 1 and readjust if necessary.

- Securely tighten the two chain regulator lock nut 2.

- Tighten the rear torque connection nut (rear) to the specified torque.

Caution

- The chain should be through the back of the chain, the lock inserted outside, the opening of the lock should be in the opposite direction of rotation.

- Keep the chain clean and lubricated, so that the chain works normally and extend the service life of the chain.

- When replacing the transmission chain, please replace the transmission chain and sprocket sets.

- Rear torque link nut: 13 N.m (1.3 kgf-m)

- Rear wheel nut: 65 N.m (6.5 kgf.m)

15—Schematic diagram of the braking system:

15.1—Precautions in operation：

Caution

- Inhaling asbestos may cause disorders of respiration system or cancer, therefore, never use air hose or dry brush to clean brake parts. Use vacuum cleaner or other authorized tool instead.

- The brake caliper can be removed without removing the hydraulic system；

- After the hydraulic system is removed, or the brake system is felt to be too soft, bleed the hydraulic system；

- While refilling brake fluid, care should be taken not to let the foreign material entering into the brake system；

- Do not spill brake fluid on the painted surfaces, plastic or rubber parts to avoid damage；

- Check the operation of the brake system before riding。

Specifications                                                            unit: mm

| Item | Standard | Limit |
| --- | --- | --- |
| The thickness of front brake disk | 4.000 | 2.500 |
| The thickness of rear brake disk | 4.000 | 2.500 |
| Front and rear brake disk eccentricity | < 0.100 | 0.300 |
| Front brake master cylinder inner diameter | 12.7+0.055 0 | 12.765 |
| Front brake master cylinder piston outer diameter | 340 -0.03 | 33.965 |
| Rear brake master cylinder inner diameter | 12.7+0.055 0 | 12.765 |
| Rear brake master cylinder piston outer diameter | 250 -0.03 | 24.965 |
| Diameter of front disk | 240.00 | / |
| Diameter of rear disk | 240.00 | / |
| Thickness of front brake lining | 10.000 | 4.000 |
| Thickness of rear brake lining | 5.000 | 2.000 |

Torque values:

Brake hose bolts：	3.0~4.0kgf.m

Bolt for front brake caliper：	2.9~3.5kgf.m

Brake lever nut：	0.8~1.0kgf.m

Air-bleed valve：	0.8~1.0kgf.m

15.2—Troubleshooting:

Soft brake lever

- Air inside the hydraulic system

- Hydraulic system leaking

- Worn master piston

- Worn brake pad

- Poor brake caliper

- Worn brake lining/disk

- Low brake fluid

- Blocked brake hose

- Warp/bent brake disk

- Bent brake lever

Hard operation of brake lever

- Blocked brake system

- Poor brake caliper

- Blocked brake pipe

- Seized/worn master cylinder piston

- Bent brake lever

### Uneven brake

- Dirty brake lining/disk

- Poor wheel alignment

- Clogged brake hose

- Deformed or warped brake disk

- Restricted brake hose and fittings

Tight brake

- Dirty brake lining/disk

- Poor wheel alignment

- Deformed or warped brake disk

Brake noise

- Dirty lining

- Deformed brake disk

- Poor brake caliper installation

- Imbalance brake disk or wheel

15.3-Disk brake system inspection：

- By visual examination whether divulges or the damage, with spanner inspection brake tube seam whether becomes less crowded, and the inspection handle bar turn right or turn left, or pressure the cushion, whether besides the pipeline protection department, whether there is interferes, contacts other parts of；

Brake linings wear limit grooves

- Remove the front brake pad cap；

- Check the brake from behind the brake caliper；

Brake caliper	Brake disk

The brake pad must be replaced with new lining when the brake pad wear limit reaches the brake disk；

- Park the motorcycle on a level ground, and check if fluid level is under the “LOWER” mark；

- Recommended Brake Fluid: WELL RUN BRAKE OIL (DOT 3).

peep door

- The vehicles inclined or just stop, the survey oil level could not be accurate, had to settle the 3~5 minute；

- In order to prevent has the chemical change, please do not use counterfeiting or other unclear trade marks brake fluid；

- Use standard specification brake fluid to ensure braking efficiency.

Caution

15.4—Add brake fluid：

Before the brake fluid reservoir is removed, turn the handle so that the brake fluid reservoir becomes horizontal, and then remove the brake fluid reservoir.

Front brake master cylinder

When maintenance brake system, will be supposed to paint the surface or the rubber parts catches up by the rags.

- Supplement brake fluid please do not surpass the upper limit, spilled brake fluid on painted surfaces, plastic or rubber components may result in their damages.

Caution

Rear brake master cylinder

Remove the master cylinder cap and diaphragm；

- Increases the high quality brake fluid, uses by all means must with the trade mark brake fluid joins in the master cylinder；

- Clean the dirty brake disk。

Caution

- The dirty brake lining or disk will reduce the brake performance.

- To mixed non-compatible brake fluid will reduce brake performance.

- Foreign materials will block the system causing brake performance to be reduced or totally lost.

Air-bleed valve

15.5-Brake fluid replacement/Air-bleed:

- 1.Connect drain hose to air-bleed valve；

- 2.Open the drain valve on the calipers and delay valve the brake lever until the old brake fluid is entirely drained out；

Front brake caliper

3.Close the drain valve and add specified brake fluid into the brake master cylinder。

Air-bleed valve

Recommended brake fluid:

WELLRUN DOT 3 brake fluid

- 4.Connect one end of transparent hose to the drain valve, and put the other end into a container；

Rear brake calipersr

Transparent hose

Air-bleed valve

Bubble

5.Open the drain valve around 1/4 turns, and at the same time hold the brake lever until the there is no air bubble in the drain hose and also feeling resistance on the brake lever；

- 6.Close the drain valve when finishing the brake system refilling fluid procedure, and operate the brake lever to check whether air bubble is in brake system or not；

- 7.If brake is still soft, please bleed the。

- 7.1.Tightly hold the brake lever and open the drain valve around 1/4 turns, and then close the valve；

- Discharge air from the lower pump;

- The brake fluid can be replaced by machine, with shorter time and less bubbles.

- Do not release the brake lever before the drain valve is closed.

- Always check the brake fluid level when carrying out the air bleeding procedure to avoid air enter into the system.

7.2.Slowly release the brake lever, and wait for       Caution

- a few seconds until it reaches its top

- position；

- 7.3.Repeat the steps 1 and 2 until there is no air bubble at the end of the hose；

- 7.4.Tightly close the drain valve；

- 7.5.Make sure the brake fluid is in the UPPER level of the master cylinder, and refill the fluid if necessary；

- 7.6.Cover the cap.

15.6-Front brake caliper：

# Removal

Caliper mounting bolts

Brake hose bolts

Place a container under the brake caliper, and loosen the brake hose bolts and finally remove the brake hoses；

Caution

Caliper mounting bolts

Do not spill brake fluid on painted surfaces.

Cotter pins

Remove two caliper mounting bolts and the caliper。

Installation

- Install the brake caliper and tighten the mounting bolts。

Torque: 2.9~3.5kgf.m

Caution

Use M8x35 mm flange bolt only.

- Long bolt will impair the operation of brake disk.

- Use two seal washers and hose bolts to lock the hoses and brake caliper in place.

Torque: 3.0~4.0kgf.m

Elastic reed

Refill up the brake fluid to the reservoir and make necessary air bleeding.

Brake pad replacement

- Remove brake caliper;

- Remove brake pads;

- Install the Elastic reed;

Brake pad

Install the Elastic reed,new brake pads onto brake caliper;

- tighten the Cotter pins;

15.7-Rear Brake Caliper：

Brake hose bolts

Removal

- Place a container under the brake caliper, and loosen the brake hose bolts and finally remove the brake hoses.

Caution

- Do not spill brake fluid on painted surfaces.

- Remove two caliper mounting bolts and the caliper.

Caliper mounting bolts

Installation

Install the brake caliper and tighten the mounting bolts.

Torque: 2.9~3.5kgf.m

Caution

Cotter pins

Use M8 x 35 mm flange bolt only.

- Long bolt will impair the operation of brake disk.

Brake pad

Use two seal washers and hose bolts to lock the hoses and brake caliper in place.

Torque: 3.0~4.0kgf.m

- Refill up the brake fluid to the reservoir and make necessary air bleeding.

Brake pad replacement

- Remove brake caliper;

- Remove brake pads;

- Install the Elastic reed;

- Install the Elastic reed,new brake pads onto brake caliper;

- tighten the Cotter pins;

- Install the cover plate and tighten the cross screws;

Brake disk

Micrometer

15.8—Brake Disk：

Inspection

- Visually check the brake disk for wear or break.

- Measure the thickness of the disk at several places.

- Replace the disk if it has exceeded the service limit.

- Allowable limit:

- Front brake disk	2.5mm

- Rear brake disk	2.5mm

Brake hose

Brake switch

Remove the brake disk from wheel.

- Check the disk for deformation and bend.

- Allowable limit: 0.30 mm

- The dirty brake lining or disk will reduce the brake performance.

- Brake lining includes the asbestos ingredient, cannot use the air-gun to be clean, the operator should dress the mouthpiece and the glove, use vacuum cleaner clean it.

Caution

15.9—Master Cylinder：

Master Cylinder Removal

Caution

- Do not let foreign materials enter into the cylinder.

2 bolts

Caution

- The whole set of master cylinder, piston, spring, diaphragm and cir clip should be replaced as a set.

- Remove the handlebar covers；

- Remove the leads of brake light switch；

- Drain out the brake fluid；

Master cylinder

Spring

Piston

Piston

Cir clip

Rubber boot

Remove the brake lever from the brake master cylinder；

- Remove the brake hose；

- Remove the master cylinder bolts and the master cylinder。

- Remove the rubber pad；

- Remove the cir clip；

- Remove the piston and the spring；

- Clean the master cylinder with recommended brake fluid。

Master cylinder

Master Cylinder Inspection：

- Check the master cylinder for damage or scratch， Replace it if necessary；

- Measure the cylinder inner diameter at several points along both X and Y directions；

- Replace the cylinder if the measured values exceed allowable limit。

- Allowable limit:

- Front brake:12.755 mm

- Rear brake: 12.755 mm

Measure the outer diameter of the piston：

Master cylinder piston

Replace the piston if its measured value exceeds allowable limit。

- Allowable limit:

Front brake: 12.645 mm

Rear brake : 12.645 mm

Master Cylinder Assembly    Caution

- It is necessary to replace the whole set comprising piston, spring, piston cup, and cir clip.

- Make sure there is no dust on all components before assembling.

Master cylinder

Spring

Piston

Piston

Cir clip

Rubber boot

Apply clean brake fluid to the piston cup, and then install the cup onto the piston；

- Install the larger end of the spring onto the master cylinder；

- The master cup’s cavity should be face inside of master cylinder when installing the master cup；

- Install the cir clip。

Caution

- Never install cup lip in the opposite direction.

- Make sure the cir clip is seated securely in the groove.

2 bolts

Master Cylinder Install：

- Install the rubber pad into the groove correctly；

- Place the master cylinder onto handlebar, and install the bolts；

- Install the brake lever, and connect leads to brake light switch。

- Connect brake hoses with 2 new washers；

Washers

Brake light switch

Brake hose

Tighten the brake hose bolt to the specified torque value；

- Make sure the hose is installed correctly；

- Install all wires, hoses, and components carefully so avoid to twisting them together.

- Kink of brake leads, hose or pipe may reduce brake performance.

- Improper routing may damage leads, hoses or pipes.

Caution

Add specified brake fluid and bleed the system

# III. Engine maintenance

## 16-Main characteristics:

1.Using manual clutch, gear shift speed change system, simple and reliable operation.

2.Cam mechanism is used to control air intake and exhaust to give full play to mechanical properties.

3.Using ECU electric control ignition, with good starting performance.

4.Using the nozzle for oil, with good atomization, small fuel consumption.

5.Large power, torque, stable power operation, low noise.

6.Easy maintenance, safe and reliable work.

## 17-Main technical specification:

1.Model: K157FMI;

2.Type: four-stroke, single-cylinder, vertical, forced air cooling;

3.Main performance parameters:

3.1.Maximum power and corresponding speed: 7.5KW/9000(r/min)

3.2.Maximum torque and corresponding speed: 8.2N .m/7000(r/min)

3.3.Minimum fuel consumption: 350g/KW.h

3.4.Minimum no-load stable speed:1700±100 (r/min)

4.Main structural parameters:

4.1.Cylinder diameter:57㎜

4.2.Piston stroke:48.8㎜

4.3.Working volume of cylinder:124.5mL

4.4.Compression ratio: 9.5:1

4.5.Ignition mode: ECU electric control ignition

4.6.Ignition advance Angle: Front of the piston stop point 15°±2°/ (1500±100) r/min-35°±2°/ (4000±100) r/min。

4.7.Starting mode: electric starting

4.8.Operation mode: pressure, splash and use

4.9.The direction of rotation of the engine: facing the left end of the crankshaft counterclockwise

5.Transmission device:

5.1.Type of transmission: V-belt automatic centrifugal stepless transmission at the front and gear two-stage transmission at the rear;

5.2.Reduction ratio:

| 5 File | 3.000 |
| --- | --- |
| 4 File | 1.857 |
| 3 File | 1.368 |
| 2 File | 1.095 |
| 1 File | 0.913 |

5.3.Clutch form: Wet-type multi-plate friction-type clutch

6.Supporting accessories:

6.1. Oil supply type:Electric injector

6.2.Spark plug: CR7HSA

6.3.Air filter type: soft polyurethane foam plastic filter element

6.4.Magneto Model: /

6.5.Starting motor: DC permanent magnet type

6.6.Oil pump: rotor type

7.Fuel & Lubricants

7.1.Gasoline grade :90 and above unleaded

7.2. Lubricating oil grade:

Crankcase:SAE grade 15W/40

7.3. Oil capacity (oil change):

crankcase :0.85L

- 8.Overall dimension:

- long×wide×Height: (416×290×390)㎜

- 9.Net mass: 33Kg

18-Precautions for use:

1.For electric starting, press the start button for no more than 15S each time, and the interval should be more than 10S. After the engine starts, release the starting button immediately to prevent damage to the engine.

2.In order to extend the service life of the engine, it must be preheated for 1-3 minutes after each start.

3.Before starting, check whether the parts are loose or defective, and whether the throttle handle, brake and other control mechanisms are flexible and reliable.

4.The running-in mileage of the new machine is 300km, the speed during the running-in period should not be more than 70km / h, and the engine oil should be replaced in time. After the engine is running-in well, it is conducive to the performance of the engine and the extension of its service life.

5.Addition and replacement of lubricating oil:

5.1.Add: The user should often check the amount of crankcase oil, the oil scale is inserted into the oil inspection hole, the oil level should be between the upper and lower scale, lower than the bottom line or close to the bottom line should be timely added oil.

5.2.Replacement: Idle the engine for 3 to 5 minutes, so that the engine heating, oil temperature rise easy to flow. After stopping, remove the oil bolt under the right crankcase cover (hexahexagonal width is 17㎜), the dirty oil is discharged, the oil filter net is cleaned and installed back, the new oil is re-added from the oil scale hole, and the oil scale is tightened after confirming the oil capacity.

6.Air filter cleaning:

Remove the filter element, first pat the dust on the outside of the paper filter element, and then use the air gun to blow the filter element clean. The dusty filter element must be replaced. Motorcycles driving on dusty roads should increase the frequency of cleaning and replacement of air filter elements.

7.Adjustment of valve clearance:

7.1.The valve clearance (hot state) of the K157FMI engine is specified as follows:

Valve clearance: 0.05～0.08㎜.

7.2.Adjustment steps:

7.2.1.First, find the top dead center of piston compression stroke, remove the rubber plug on the fan cover, and observe the scale line on the magneto;

7.2.2.Then remove the spark plug on the cylinder head (using a special sleeve), turn the fan impeller clockwise with the sleeve, and observe that when the top dead center mark is aligned with the marking of the magneto flywheel shell, the piston is in the top dead center position;

7.2.3.At this time, there should be valve clearance, and the piston is in the compression top dead center position. Otherwise, you need to rotate the crankshaft for another week, and then adjust the valve clearance as required, and lock the valve clearance adjustment nut. The valve clearance should be detected by a gauge, and it cannot be detected by hand.

19-Regular maintenance project and schedule:

| - Project | - Driving distance（KM） |  |  |  |  |  |  | - Often |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | - 300 | - 1000 | - 3000 | - 6000 | - 9000 | - 12000 | - 15000 |  |
| - Crankcase oil |  |  | - 2000Km/time |  |  |  |  | - ☆ |
| - Gearbox oil |  |  |  |  |  |  |  |  |
| - Air filter element |  |  | - △ | - △ | - △ | - △/ | - △ | - In dusty areas, clean once every 1000km |
| - Valve clearance |  | - ☆ |  | - ☆ |  | - ☆ |  |  |
| - Spark plug |  |  | - ☆ | - ☆/ | - ☆ | - ☆/ | - ☆ |  |
| - throttle valve body |  |  |  | - △ |  | - △ |  |  |
| - Carbon deposition |  |  |  |  |  | - △ |  |  |

- ☆:Inspect  △:Clean  :Replace